崇新学堂

2025－2026学年第一学期

实 验 报 告

课程名称：       电子信息工程导论

实验名称：     Homework 2 Heads Up

专  业  班  级            崇新学堂

学  生  姓  名  高子轩，钱竹玉，吕思洁，徐亚骐

实  验  时  间          2025年10月28日

Step 1

Solving steps:

With the input e[n] ,we need to get vs[n] .

Voltage vs[n] is proportional to the angle between the light’s angular position and that of the head

vs[n]=kse[n]

Voltage vc[n] is proportional to vs[n]

vc[n]=kcvs[n]

We can get the result as follow

vs[n]=kckse[n]

System function Hcs

Hcs=VcE=kcks

The complete system block diagram is as follow

Figure 1 the diagram for the control/sensor system

Step 2

We define a function controllerAndSensorModel, which takes as input a gain kc, and outputs an instance of sf.SystemFunction which represents the controller/sensor system with this gain.

Here is the key code

def controllerAndSensorModel(k_c):

return(sf.Gain(k_s*k_c))

Step 3

At step n, θh[n] is equal to θh[n-1] from step n-1 plus the displacement Tωh[n-1], we get

θh[n]=θh[n-1]+Tωh[n-1]

For convenience, we use the R operator to simplify calculations, so we get

Θh=RΘh+TRΩh

After transposition, we can get Hint

Hint=ΘhΩh=TR1-R

The complete system block diagram is as follow

Figure 2 the system function for the integrator

Step 4

We define a method in integrator, which takes as input a timestep T (the length of time between samples) and returns an instance of sf.SystemFunction which represents an integrator with the appropriate timestep.

Here is the key code

def integrator(T):

accumulator = sf.FeedbackAdd(sf.R(), sf.Gain(1))

return sf.Cascade(sf.Gain(T), accumulator)

Step 5

With the input vcn ,we need to get ωh[n]

Through the preconditions, we can get

αh[n]=kmim[n]

imn=vcn-vbnvcnrm

vbn=kbωhn

At step n,ωh[n] is equal to ωh[n-1] from step n-1 plus the displacement Tαh[n-1], we get

ωh[n]=ωh[n-1]+Tαh[n-1]

For convenience, we use the R operator to simplify calculations, so we get

Ωh=RΩh+TRAh

Substitute the equation related to Ah,we get

Ωh=RΩh+TRkmIm

Substitute the equation related to Im,we get

Ωh=RΩh+TRkmVc-Vbrm

Substitute the equation related to Vb,we get

Ωh1-R+TRkmkbrm=TRkmrmVc

Then we get Hmotor

Hmotor=ΩhVc=TkmrmR1-R+TkmkbrmR

The complete system block diagram is as follow

Figure 3 the system function for the motor

Step 6

We define a method motorModel, which takes the same input as integrator, but returns an instance of sf.SystemFunction which represents the motor.

Here is the key code

def motorModel(T):

alpha = T * k_m / r_m

beta = T * k_m * k_b / r_m

H_loop = sf.FeedbackAdd(sf.R(), sf.Gain(1 - beta))

return sf.Cascade(sf.Gain(alpha), H_loop)

Step7

With the input vcn,we need to get θh[n]

From motor, we get a system whose input is vcn and whose output is ωh[n]

From integrator, we get a system whose input is ωh[n] and whose output is θh[n]

Then all we need to do is cascade these two systems.

The complete system block diagram is as follow

Figure 4 the system function for the plant

We can easily get

Hplant=HmotorHint=T2kmrmR2(1-R)1-R+TkmkbrmR

Step 8

We define a method plantModel, which takes the same input as integrator and motorModel, and returns an instance of sf.SystemFunction which represents the entire plant.

Here is the key code

def plantModel(T):

return sf.Cascade(motorModel(T),integrator(T))

Step 9

With the input θl[n],we need to get θh[n]

From Controllers, we get a system whose input is e[n] and whose output is vcn. Among that, e[n]=θl[n]-θh[n].

Part of e[n] comes from the input θl[n] ,so we cascade the Controllers and the Plant. Then we can get a part of the output.

Another part is θh[n] , so we should construct a degenerative feedback road.

The complete system block diagram is as follow

Figure 5 the system function for the composite system

The system function of forward path G

G=HcsHplant=(kcks)T2kmrmR2(1-R)1-R+TkmkbrmR

The system function of the feedback path Hf=1

The whole system function H

H=G1+GHf

After simplification, the system function H

H= T2kmkcksrmR21+Tkmkbrm-2R+1-Tkmkbrm+TkmkcksrmR2

Step 10

To solve for the pole P, we set the denominator to 0.That is to say

1+Tkmkbrm-2R+1-Tkmkbrm+TkmkcksrmR2 = 0

Using the root formula x=-b±b2-4ac2a and replacing Tkmkbrm, Tkmkcksrm,R with β, γ, 1z , we get

z2+(β-2)z+(1-β+γ)=0

After solving this equation, we get

z=2-β±β2-4γ2

R is the final answer we need, so replace z with 1R .Then we get

p=2-Tkmkbrm±2-Tkmkbrm2-41-Tkmkbrm+Tkmkcksrm21-Tkmkbrm+Tkmkcksrm

Step 11

We define a method lightTrackerModel, which takes as input the gain to use for the controller (kc) and the length of a timestep (T), and returns an instance of sf.SystemFunction which represents the entire light-tracking system with the specified gains and timesteps.

Here is the key code

def lightTrackerModel(T,k_c):

forward_path = sf.Cascade(controllerAndSensorModel(k_c), plantModel(T))

return sf.FeedbackSubtract(forward_path,sf.Gain(1))

Step 12

Find the fastest convergence and no oscillation, that is, find the kc that minimizes the amplitude of the dominant pole.

We use the procedure optimize.optOverLine to help us search for the best k. We set numXsteps = 1000 and change the xmin, xmax

| times | xmin | xmax | kc |
| --- | --- | --- | --- |
| 1 | 0 | 100 | 0.6000 |
| 2 | 0 | 25 | 0.6000 |
| 3 | 0 | 1 | 0.6240 |
| 4 | 0 | 0.7 | 0.6244 |
| 5 | 0 | 0.65 | 0.6247 |

Table 1 the result of procedure optimize.optOverLine

We find the kc approaches 0.625. Then 0.625 is the best answer.

Figure 6 the situation when k=0.625

Step 13

The system is

monotonically convergent when p is a real number;

divergent when the amplitude of p exceeds 1;

oscillating convergent when p has an imaginary part but the amplitude does not exceed 1.

Judgment basis

Not oscillating means Δ greater than or equal to zero. We solved that

k<0.625

Non-divergence means the |p| < 1 .We solved that

k<20

The final result

| k | Convergence or divergence |
| --- | --- |
| 0<k<0.625 | monotone convergence |
| 0.625<k<20 | Oscillating Convergence |
| k>20 | divergence |

Table 2 The final result of k’s range

Figure 7 the situation when kc=0.5(<0.625)

Figure 8 the situation when kc=2(>0.625)

Figure 9 the situation when kc=25(>20)

Step 14

We found that when k=0.625 is fixed, increasing T alters the convergence characteristics; for example, oscillations appear as shown in the figures below.

Figure 10 the situation when T=0.1

Although the number of convergence steps appears smaller, the actual time is longer because the time scale has changed. When T decreases, the poles move closer to the unit circle, resulting in a slower convergence rate.

Figure 11 the situation when T=0.004

Appendix: The Description of AI Usage in the Report

During the task process, we relied on AI to help us understand the relevant concepts and problems. In the debugging of parameters, we utilized AI to efficiently achieve the goal Due to language issues, we also employed AI translation to help us quickly understand the task objectives